# Health and safety checklist

## Class B Asbestos (non-friable) removal work

Note:  non-friable means that the Asbestos Containing Material (ACM) is not in powder form, and when dry does not crumble by hand pressure

Work Health and Safety Act 2020 Work Health and Safety (General) Regulations 2022 and Work Health and Safety (Mines) Regulations 2022 This checklist is a summary of asbestos specific requirements in the WA work health and safety laws. It does not include all legislative requirements and does not address general safety requirements.

| Completed by |  |
| --- | --- |
| Full name | enter full name |
| Company name | enter company name |
| Site address Suburb Postcode | enter site address enter suburb enter postcode |
| Date | enter date DD/MM/YYYY |
| Telephone | enter telephone |

| Check | WHS Act / Regulation | Yes | No | N/A | Notes / comments |
| --- | --- | --- | --- | --- | --- |
| Planning and preparation |  |  |  |  |  |
| Is an asbestos removal licence required? Note: required if more than 10 m2 | rr. 458, 487 | ☐ | ☐ | ☐ | enter notes/comments |
| If a licence is required, is a copy of the licence available on site? | r. 505 | ☐ | ☐ | ☐ | enter notes/comments |
| Has the regulator been notified of licensed removal work at least 5 days in advance? https://wise.commerce.wa.gov.au/wise-online/nar Note: In an emergency, call 1300 307 877 to notify (during business hours) and notify online within 24 hours | r. 466 | ☐ | ☐ | ☐ | enter notes/comments |
| Has the scope of work been documented with the client? i.e. what is being removed, what is being left, any asbestos debris already present, access to water, what area will be restricted? | Recommended | ☐ | ☐ | ☐ | enter notes/comments |
| Have certain persons been advised of removal works? Note: person with management or control of workplace, and for residential - those in immediate vicinity e.g. adjoining properties, occupier, owner. | r. 467 | ☐ | ☐ | ☐ | enter notes/comments |
| Have a safe work method statement (SWMS) and an asbestos removal control plan (ARCP) been written? (after site inspection to include site specific hazards) Note: It is OK to combine these into one document provided all required information is included | rr. 299, 464 | ☐ | ☐ | ☐ | enter notes/comments |
| Have all workers conducting licensed asbestos removal passed the VET course? Note: remove non-friable asbestos (phased in requirement – by 31/03/2024) | r. 460 | ☐ | ☐ | ☐ | enter notes/comments |
| Has the nominated supervisor passed the specified VET courses? Note: remove Non-Friable Asbestos and Supervise Asbestos Removal (phased in requirement – by 31/03/2024) | r. 460 | ☐ | ☐ | ☐ | enter notes/comments |
| Health monitoring program in place if – - Asbestos removal work is conducted regularly (e.g. weekly or more often) and/or - There is potential for higher levels of exposure (e.g. incorrect work practices/use of PPE, workers not clean shaven etc.) |  | ☐ | ☐ | ☐ | enter notes/comments |
| Has the asbestos register been consulted where available? | r. 463 | ☐ | ☐ | ☐ | enter notes/comments |
| If there is no asbestos register, or if the asbestos register has limitations, has asbestos been identified by a competent person? | s.19, r. 451 | ☐ | ☐ | ☐ | enter notes/comments |
| Have the asbestos removal boundaries been established and barriers used where practicable? | r. 469 | ☐ | ☐ | ☐ | enter notes/comments |
| Have entry points to the asbestos work area been signposted? | r. 470 | ☐ | ☐ | ☐ | enter notes/comments |
| Have electrical and lighting installations in the asbestos removal area been disconnected, removed or de-energised by a licensed electrician? | s. 19, r. 152 | ☐ | ☐ | ☐ | enter notes/comments |
| Are hand tools either non-powered or low-powered and designed to capture or suppress dust? Note: No high speed tools – grinders, sanders, saws | s.19, r. 446 | ☐ | ☐ | ☐ | enter notes/comments |
| Is suitable PPE available? Note: Minimum – coveralls, Class P2 respirator, safety boots, gloves where assessed as necessary? | r. 44 | ☐ | ☐ | ☐ | enter notes/comments |
| Are workers who are required to wear a fitted Class P2 respirator (disposable or reusable) clean shaven? | rr. 44, 46 | ☐ | ☐ | ☐ | enter notes/comments |
| Does the asbestos vacuum cleaner comply with AS/NZS 60335.2.69 Industrial vacuum cleaners and have a Class H filter? Note: No domestic vacuum cleaners used or brooms/brushes where it is practicable to use a HEPA vacuum | s.19, r. 446 | ☐ | ☐ | ☐ | enter notes/comments |
| Has the work area been prepared by removing all items as far as practicable and covering any remaining items with heavy duty plastic sheeting? | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| Are drop sheets used where practicable? | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| Has an area for wrapping waste ACM been prepared and heavy duty sheeting (200µm minimum thickness) laid out or heavy duty asbestos waste bags provided? | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| For indoor work, is air conditioning turned off and openings to other parts of the building sealed? | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| For outdoor work, are openings to buildings closed where practicable? | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| Prior to removal, has the ACM been saturated by water containing a wetting agent (e.g. detergent), via a low pressure spray? OR has a tinted PVA coating been applied? | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| Is run-off of the wetting agent minimised to avoid contamination issues? | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| Has all of the equipment being used for the asbestos removal been inspected before commencing removal work? | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| Where air monitoring has been identified as appropriate, has a competent person, independent from the removalist, been selected to conduct air monitoring? | r. 482 | ☐ | ☐ | ☐ | enter notes/comments |
| Asbestos removal |  |  |  |  |  |
| Is the nominated supervisor on site or readily available to attend site? | r. 459 | ☐ | ☐ | ☐ | enter notes/comments |
| Has asbestos been removed before demolition of the structure, so far as practicable? | rr. 452, 453 | ☐ | ☐ | ☐ | enter notes/comments |
| Is controlled wetting of the ACM continuing during removal (for dust suppression)? | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| If a wet method of removal cannot be used (due to live electrical conductors or roof work) have the following controls been implemented or considered by a risk assessment? - PVA coating (tinted to show coverage) prior to removal; - ACM removed in small sections with limited disturbance; - work area is enclosed and under negative pressure; - all personnel involved in the removal are using full-face positive pressure air supplied respirators; - waste material is placed immediately into wetted containers. | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| Is removal conducted with minimal breakage of ACM? | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| Has a waste disposal plan been developed to include safe transport to the waste disposal site and any specific requirements of the disposal facility? | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| Has asbestos waste been wrapped/contained until it can be removed and disposed of? | r. 472 | ☐ | ☐ | ☐ | enter notes/comments |
| Are waste bags tied off securely, and waste packs fully sealed with heavy duty tape? | s. 19, r. 472 | ☐ | ☐ | ☐ | enter notes/comments |
| Are the waste packs or bags labelled with an appropriate warning? (e.g. CAUTION – ASBESTOS - MAY CAUSE CANCER) | r. 472 (Sch. 9 cl. 8) | ☐ | ☐ | ☐ | enter notes/comments |
| Are records of disposal kept? | Licence condition | ☐ | ☐ | ☐ | enter notes/comments |
| Has a visual check been conducted prior to decontamination to ensure there is no visual evidence of dust or debris? Note: check ledges, tops of duct work, crevices and cracks in the floor/wall |  | ☐ | ☐ | ☐ | enter notes/comments |
| Decontamination |  |  |  |  |  |
| Has the asbestos removalist provided suitable facilities for decontamination of the removal area, the equipment and the workers? | r. 471 | ☐ | ☐ | ☐ | enter notes/comments |
| Has the workplace been decontaminated using a wet method or HEPA vacuum? | r. 471 | ☐ | ☐ | ☐ | enter notes/comments |
| Have all tools used during the asbestos removal process been decontaminated? Note: if tools cannot be decontaminated or are to be used for other ACM removal, they should be tagged, double bagged and correctly labelled | r. 471 | ☐ | ☐ | ☐ | enter notes/comments |
| Has disposable asbestos contaminated PPE been double bagged for disposal? | r. 472 | ☐ | ☐ | ☐ | enter notes/comments |
| Has footwear worn during asbestos removal been thoroughly cleaned? Note: use of an asbestos vacuum and/or wet wiped | r. 472 | ☐ | ☐ | ☐ | enter notes/comments |
| Has respiratory protective equipment been used until all contaminated clothing has been thoroughly vacuumed or disposed of? | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| Has personal decontamination been conducted in the asbestos work area to avoid re-contamination? Note: minimum – wash face, arms, and hands and shower if available | s. 19, r 471 | ☐ | ☐ | ☐ | enter notes/comments |
| Soil contamination |  |  |  |  |  |
| Has the removal area and work area been carefully visually inspected and any remnant ACM collected for disposal? | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| Will hand (emu) picking and/or raking remove remnant ACM? Note: if not, removal of the top soil may be necessary | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| Has soil been dampened down before hand picking of ACM or raking occurs? | s. 19 | ☐ | ☐ | ☐ | enter notes/comments |
| Is appropriate PPE being worn while handling waste ACM? | rr. 44, 46 | ☐ | ☐ | ☐ | enter notes/comments |
| Has the ACM and disposable PPE been double bagged and disposed of as asbestos waste? | rr. 471, 472 | ☐ | ☐ | ☐ | enter notes/comments |
| Have decontamination procedures (as above) been followed? | s. 19, r. 471 | ☐ | ☐ | ☐ | enter notes/comments |
| Clearance |  |  |  |  |  |
| For asbestos removal at a workplace, has the person commissioning the removal provided a clearance inspection by an independent competent person? | r. 473 | ☐ | ☐ | ☐ | enter notes/comments |
| For asbestos removal at a residence, has the asbestos removalist ensured that a clearance inspection has been done by an independent competent person? | r. 473 | ☐ | ☐ | ☐ | enter notes/comments |
| Has a joint walk through the removal area with the client been conducted at the end of the job, to check both agree the area is visually clean? | Recommendation | ☐ | ☐ | ☐ | enter notes/comments |
| Notes |  |  |  |  |  |

## Other sources of information

- https://www.wa.gov.au/organisation/department-of-energy-mines-industry-regulation-and-safety/worksafe

- Work Health and Safety Act 2020

- Work Health and Safety (General) Regulations 2022

- Work Health and Safety (Mines) Regulations 2022

- DEMIRS licensing – Asbestos licences

- Notify the regulator of licensed asbestos removal

- Notify the regulator of a workplace injury or dangerous incident

A43253611